Research Analysis: Chicken early disease detection via Vocalization Signals

1. Overview and Objective

- The primary goal of this research is to address the gap in publicly available auditory datasets for early disease detection in chickens, specifically regarding respiratory illnesses.

- The study aims to facilitate the development of non-invasive, automated "smart technologies" that can monitor chicken health continuously.

2. Experimental Design and Data Collection

The researchers conducted a controlled experiment at the Bowen University Poultry Research Farm:

- Subjects: 100 day-old chickens were purchased and split into two groups.

- Methodology: For respiratory illnesses, the first group received treatment, whereas the second group did not.

- Monitoring: The chickens were separated and caged in a monitored environment.

- Observation: After 30 days, the untreated group started to sound sick with respiratory issues, exhibiting symptoms like coughing and snoring.

3. Audio Data Acquisition

- Recording Setup: Microphones were set a reasonable distance away from the chickens to eliminate extraneous sounds and background noise.

- Schedule: Audio data was continually collected for 65 days, three times per day (morning, afternoon, and night).

- Technical Specs: The data was gathered using 24-bit samples at 96 kHz.

- Dataset Composition: The final dataset contains 346 audio signal files organized into three folders:

  - Healthy: 130-139 audio files.

  - Unhealthy (Sick): 110-121 audio files.

  - Noisy: 86 audio files, including background noises like moving vehicles and human voices.

4. Technical Signal Processing

To make the raw audio suitable for machine learning, the researchers employed specific signal processing techniques:

- Conversion: Chicken audio signals were recorded in MA4 and converted to WAV format.

- Framing: Nonstationary sound samples were framed by a shifting Hamming window to create 10 to 30-second stationary signals.

- Noise Reduction: A Kalman filter—a nonstationary, recursive filter—was used to improve the signal-to-noise ratios of the acoustic samples.

- Spectral Analysis: In the frequency domain, normal chicken sounds have frequency content around the y-axis, while anomalous sounds exhibit a noticeable spike of about 0.2 radians.

5. Research Value and Applications

- Early Detection: Audio signal datasets of chicken vocalization have potential value in early diagnosis due to the high contagion rate of respiratory diseases.

- FAIR Data: Making this annotated dataset available promotes Findability, Accessibility, Interoperability, and Reproducibility (FAIR Data).

- Model Training: The dataset is applicable in developing computer vision-based models for audio signal segmentation and classification to aid in precise disease detection

Mark this point

Dataset for this project is available ..